# Project Abstract (2-3 pages)

## Title

Pipelined RV32IM RISC-V Core with L1 Cache Hierarchy and Dedicated Neural Network MAC Coprocessor

Group Number: 13

## 1. System Description

Briefly explain (≤ 3 lines):

##   What the system does: It is a custom 5-stage RISC-V processor that runs standard programs while using dedicated hardware blocks to speed up complex math and memory access.

##   What problem it solves: It prevents the processor from slowing down during heavy AI workloads by moving repetitive math to a coprocessor and reducing main memory delays using caches.

##   High-level functionality: The system reads instructions from a fast cache, manages data flow safely through a pipeline to prevent crashes, and routes heavy math directly to custom accelerators.

## 2. Use Cases

Give 2-3 examples (each ≤ 2 lines):

##   Basic AI and Pattern Recognition: Used in smart devices to recognize handwriting or voice commands. The example workload is running a simple neural network program that uses the MAC coprocessor for fast matrix math.

##   Real-Time Sensor Filtering: Used in robotics or audio equipment to clean up noisy sensor data. The example workload is a digital filter program (like an FIR filter) that relies heavily on the fast hardware multiplier.

##   Embedded Control Systems: Used to manage motors or industrial machines quickly without lagging. The example workload is a control algorithm that utilizes the hardware divider and caches to process math and memory data instantly

## 3. FPGA Relevance

Give 2-3 points (each ≤ 2 lines):

##   Custom Hardware Creation: FPGAs allow us to build custom blocks like the MAC coprocessor that standard microcontrollers lack, letting us physically test our exact design.

##   Parallel Execution: Software processes instructions one step at a time, but an FPGA runs our pipeline stages, hazard logic, and multipliers at the exact same time for massive speedups.

##   True Timing Verification: Software simulations cannot perfectly mimic physical clock delays. Running it on an FPGA proves that our pipeline stalls and memory timings actually work in the real world.

## 4. System Scope

## Minimum System (Commitment)

## Implement a 5-stage pipeline with integrated hazard detection and data forwarding.

## Add a single-cycle hardware multiplier and a direct-mapped L1 Instruction Cache.

## Build a custom 64-bit Neural Network MAC coprocessor to accelerate math operations.

## Goal System (Full Intended Functionality)

## Integrate a multi-cycle hardware divider with a dedicated pipeline stall controller.

## Implement a direct-mapped L1 Data Cache to speed up read and write operations.

## Add static branch prediction using a 32-entry Branch Target Buffer (BTB).

## Stretch Goal (Advanced Features)

## Add hardware performance counters to physically track elapsed cycles and cache misses.

## Implement the RISC-V bit-manipulation extension (Zbb) for single-cycle bitwise operations.

## 5. Design Goals

##   Latency (cycles):

## 1 clock cycle for standard instructions, MAC operations, and cache hits.

## 32 to 34 clock cycles for the hardware division operation.

## 1-cycle penalty (pipeline stall) for load-use hazards.

## 2-cycle penalty (pipeline flush) for branch mispredictions.

##   Throughput:

## Achieve a peak execution rate of 1.0 IPC (Instructions Per Cycle) during continuous, hazard-free execution.

## Target a stable operating clock frequency of at least 50 MHz on the FPGA board.

##   Resource usage:

## LUTs: Less than 4,500 LUTs for the core processor, hazard logic, and divider state machines.

## BRAM: Maximum of 16 Block RAM tiles allocated for the L1 I-Cache, L1 D-Cache, and main memory.

## DSP Slices: Maximum of 4 DSP slices dedicated to the hardware multiplier and MAC coprocessor.

##   Correctness (edge cases):

## 100% pass rate on standard RISC-V (RV32IM) assembly testing programs.

## Safely handle edge cases: flawless data forwarding on back-to-back hazards, correct output for division by zero, and prevention of undetected data corruption when the 64-bit MAC accumulator reaches its maximum limit.

## 6. Implementation Approach

##   New modules: We will create dedicated Verilog modules for pipeline registers, a Hazard/Forwarding Unit, L1 Caches, a Booth Multiplier, a Divider, and the MAC coprocessor.

##   Datapath/control changes: The existing 3-stage datapath will be split into 5 stages with forwarding multiplexers added, while the Control Unit will be updated to decode custom MAC instructions and manage multi-cycle stalls.

##   Verification approach: We will use directed module-level testbenches for isolated hardware blocks, cycle-accurate Vivado RTL simulations for the full processor running assembly, and finally execute a compiled bare-metal C program (like matrix math) to prove end-to-end correctness.

## 7. Demonstration Plan

##  Input: A pre-compiled C program (like matrix multiplication) and raw data arrays loaded directly into the FPGA's block memory.

##   Processing: The custom RISC-V core will execute the code, actively using the caches and offloading the heavy math to the MAC coprocessor.

##   Output: Final calculations and cycle counts will be stored in memory, transmitted to a PC terminal via UART, and execution status shown on LEDs.

## 8. High Level Timeline

### Requirements

- Must include at least 5–6 concrete milestones

- Milestones cannot be generic like “Continue implementation”, “Work on project”

- Each milestone must produce (deliverable):

  - (code + testbench) OR FPGA demo (graded)

- At least one core functionality/module of the minimum system should be implemented and tested by Oct 25.

- Ensure minimum system (commitment) is met by Nov 1. This will leave some room to handle last-minute surprises.

| Week | Milestone |
| --- | --- |
| 5 - 11 Oct | Milestone: Base 5-Stage Pipeline Integration & Hazard Unit. Deliverable: IF-WB pipeline registers, Forwarding mux RTL + tb_pipeline_base.v and tb_hazard.v |
| 12 - 18 Oct | Milestone: Hardware Math (Multiplier/Divider) & L1 I-Cache. Deliverable: Booth RTL, Divider RTL, I-Cache RTL + tb_math.v and tb_icache.v |
| 19 - 25 Oct | Milestone: MAC Coprocessor, D-Cache & Core Integration (Oct 25 Core Deadline Met). Deliverable: MAC RTL, D-Cache RTL + integrated tb_core.v proving math/memory execution. |
| 26 Oct - 1 Nov | Milestone: Branch Prediction (BTB) & Final Full System Integration (Nov 1 Commitment Met). Deliverable: BTB RTL + Complete Minimum & Goal System FPGA Demo via UART/LEDs. |
| 2 - 6 Nov | Milestone: Buffer Week, Edge-Case Debugging & Final Documentation. Deliverable: Final project report, timing diagrams, and code cleanup. |

## 9. Team Work Plan

- Responsibilities per member

- Collaboration approach

| Member | Responsibilities |
| --- | --- |
| Pisini Ramana | 5-stage pipeline + L1-Icache +Integration |
| Chimbili Sahithi | 5-stage pipeline + Hardware Divider + Integration |
| Siddharth Misro | 5-stage pipeline + Hardware Multiplier + Mac-Coprocessor + Integration |
| Sigilipelli Praneeth Sai Bharath | 5-stage pipeline + Branch Prediction + Integration |
| Bandaru Rohith Datta | 5-stage pipeline + L1-D Cache + Integration |

## 10. Github Link

https://github.com/Rohith-Datta/RV32IM-5Stage-Core.git

-----------------------------------------------------------------------------------------------------------------------

Project Plan is Not accepted if:

- Timeline missing or milestones are generic

- No clear demo plan